在 RK3568 上，串口（UART）的驱动层级与 I2C/SPI 有很大不同。由于串口是 Linux 极其核心的设备（涉及控制台终端），开发者通常不需要像 I2C/SPI 那样编写一个从零开始的 uart_driver。

相反，RK3568 的 UART 驱动已经由 Rockchip 官方在内核中写好了（基于 8250 或 dw-apb-uart）。你通常只需要在 DTS 中开启它，然后在代码中直接调用 TTY 核心层 的接口。

## 1. 设备树描述 (DTS)

假设你使用 RK3568 的 uart3 连接一个外设（如 GPS、传感器或蓝牙），且不需要硬件流控。

&uart3 {
    status = "okay";
    pinctrl-names = "default";
    pinctrl-0 = <&uart3m0_xfer>; // 配置 TX 和 RX 引脚
    
    /* 如果外设需要特定的驱动控制（例如蓝牙芯片），可以在此添加子节点 */
    /* 如果只是通用串口通讯，通常保持空节点即可 */
};

## 2. 内核空间接口 (Kernel Space)

在内核驱动代码中，如果你需要读取串口数据，通常有两条路：

### 方法 A：通过 TTY 核心层（推荐用于外设驱动）

如果你的驱动是挂在 UART 下的子驱动（例如使用 serdev 框架），代码如下：

#include <linux/module.h>
#include <linux/serdev.h>

struct my_uart_dev {
    struct serdev_device *serdev;
};

/* 1. 接收回调：当串口收到数据时内核自动调用 */
static int my_uart_receive_buf(struct serdev_device *serdev, const unsigned char *buf, size_t count)
{
    // 处理接收到的数据 buf
    pr_info("Received %zu bytes from UART3\n", count);
    return count;
}

/* 2. 核心控制函数 */
static const struct serdev_device_ops my_serdev_ops = {
    .receive_buf = my_uart_receive_buf,
};

static int my_uart_probe(struct serdev_device *serdev)
{
    struct my_uart_dev *mdev;

    mdev = devm_kzalloc(&serdev->dev, sizeof(*mdev), GFP_KERNEL);
    if (!mdev) return -ENOMEM;

    mdev->serdev = serdev;
    serdev_device_set_client_ops(serdev, &my_serdev_ops);

    /* 打开串口并设置波特率 */
    serdev_device_open(serdev);
    serdev_device_set_baudrate(serdev, 115200);
    serdev_device_set_flow_control(serdev, false);

    /* 发送数据示例 */
    serdev_device_write(serdev, "Hello RK3568 UART", 18, 100);

    return 0;
}

static const struct of_device_id my_uart_of_match[] = {
    { .compatible = "vendor,uart_sensor", },
    { /* sentinel */ }
};

static struct serdev_device_driver my_uart_driver = {
    .driver = {
        .name = "my_uart_driver",
        .of_match_table = my_uart_of_match,
    },
    .probe = my_uart_probe,
};

module_serdev_device_driver(my_uart_driver);

## 3. 驱动层关键接口对比

| 接口类型 | 接口函数 | 特点 |
| --- | --- | --- |
| 发送数据 | serdev_device_write() | 将数据写入发送缓冲区，并支持超时设置。 |
| 接收数据 | receive_buf 回调 | 异步通知。不需要轮询，效率最高。 |
| 波特率设置 | serdev_device_set_baudrate() | 动态修改通信速率（如从 9600 切换到 115200）。 |
| 控制线路 | serdev_device_set_tiocm() | 手动控制 RTS/DTR 等引脚电平。 |

## 4. RK3568 特别说明

- 调试控制台 (FIQ Debugger)：RK3568 的 uart2 通常被内核占用作为调试控制台。如果你想把 uart2 当普通串口用，必须在 DTS 中关闭 fiq_debugger。

- DMA 模式：对于高速串口通讯（如 3Mbps 以上），建议在 DTS 中开启 dma-names = "tx", "rx";，这样可以极大降低读取数据时的 CPU 负载。

- 用户态测试：

  - 串口开启后会生成 /dev/ttyS3（对应 uart3）。

  - 可以直接使用命令测试：stty -F /dev/ttyS3 115200 && echo "test" > /dev/ttyS3。

对于你正在处理的 128 路采样数据项目，如果这些 ADC 的控制是通过串口指令下发的，使用 serdev 框架是维护性最好的方案，因为它能像 I2C/SPI 一样通过设备树管理从机。